V1 Hardware Specification

Two-tier product line for on-premises AI deployment. Pricing reflects public distributor rates as of April 2026. Includes Allodial OS and Centralized Ontology Engine — no separate software license fee.

REVISION: R4 (April 2026)   |   STATUS: Pre-pilot   |   CLASSIFICATION: Internal + Prospective Customers

| SPECIFICATION | SENTINEL (Entry) | BASTION (Mid-Market) |
| --- | --- | --- |
| TARGET MARKET |  |  |
| Org Size | 50-200 employees | 200-2,000 employees |
| Primary Use Case | Single-model inference: 7B-24B Q4, document automation, admin load-lightening | Multi-model inference: 24-48B Q4 per GPU (interactive), up to 70B Q4 batch/async (tensor parallel) |
| Network Isolation | Yes (standard on all units) | Yes (standard on all units) |
| CHASSIS |  |  |
| Form Factor | Tower workstation | 2U rackmount |
| Manufacturer | Supermicro | Supermicro |
| Model | SYS-751A-I (SuperWorkstation) | SYS-221GE-NR (GPU SuperServer) |
| Dimensions | 7.6 x 18.2 x 25.5 in | 17.2 x 3.4 x 28.5 in (2U) |
| COMPUTE - GPU |  |  |
| GPU Model | NVIDIA RTX 6000 Ada (PNY VCNRTX6000ADA-PB) | NVIDIA L40S (900-2G133-0080-000) |
| GPU Count | 1x | 2x |
| VRAM per GPU | 48 GB GDDR6 ECC | 48 GB GDDR6 |
| Total VRAM | 48 GB | 96 GB |
| CUDA Cores | 18,176 | 36,352 (2x 18,176) |
| Interconnect | PCIe Gen 4 x16 | PCIe Gen 5 x16 per GPU; no NVLink |
| FP16 Inference | 91.1 TFLOPS | ~182 TFLOPS aggregate |
| Max Model Size (FP16) | ~12B parameters | ~24B per GPU (tensor parallel) |
| Max Model Size (Q4) | ~24-30B parameters | ~70B (tensor parallel, 2x GPU) |
| Est. Throughput (Q4) | ~15-25 tok/s (24B) | ~20-30 tok/s (48B single GPU) / ~2-5 tok/s (70B tensor parallel) |
| Performance Note | Interactive inference | Interactive at 48B and below; 70B batch/async only (PCIe bandwidth limited) |
| Validation Status | Lab-tested: Pending | 48B single-GPU: Lab-tested Pending ¦ 70B tensor parallel: Lab validation pending |
| FINE-TUNING CAPABILITY |  |  |
| LoRA/QLoRA Support | Limited (7B models only, CPU offload required) | Production-ready (up to 13B native, 7B without offload) |
| Est. Fine-Tune Time | 7B LoRA on 10K samples: ~12-18 hrs (estimated) | 7B LoRA on 10K samples: ~4-8 hrs (estimated) |
| Fine-Tune Validation | Estimated from community benchmarks; lab validation pending | Estimated from community benchmarks; lab validation pending |
| COMPUTE - CPU |  |  |
| CPU Model | Intel Xeon w5-3435X | AMD EPYC 9354 (100-000000798) |
| Cores / Threads | 16C / 32T | 32C / 64T |
| Base / Boost | 3.10 / 4.70 GHz | 3.25 / 3.80 GHz |
| TDP | 270W | 280W |
| Socket | LGA 4677 | SP5 (LGA 6096) |
| MEMORY |  |  |
| Capacity | 192 GB (6x 32GB) | 512 GB (8x 64GB) |
| Type | DDR5-4800 ECC RDIMM | DDR5-4800 ECC RDIMM |
| SKU | Samsung M321R4GA0BB0-CQK (32GB) | Samsung M321R8GA0BB0-CQK (64GB) |
| Max Expandable | 256 GB (8 DIMM slots, 2 open) | 2 TB (32 DIMM slots, 24 open) |
| STORAGE |  |  |
| OS Drive | Micron 7450 PRO 960GB NVMe M.2 | Micron 7450 PRO 960GB NVMe M.2 |
| Data Drives | 2x Micron 7450 PRO 3.84TB NVMe U.3 | 4x Micron 7450 PRO 3.84TB NVMe U.3 |
| Total Raw Storage | 8.6 TB | 16.3 TB |
| Usable Storage | ~3.84 TB (ZFS mirror) | ~11.52 TB (RAIDZ1, 1-drive fault tolerance) |
| RAID Configuration | ZFS mirror (2x data drives) | ZFS RAIDZ1 (4x data drives) |
| Expansion | Up to 2x additional U.3 bays available in chassis | All 4x U.3 bays populated; expansion requires external storage shelf |
| Sentinel Storage Note | U.3 via PCIe AOC adapter (Supermicro AOC-SLG3-2E4R or equiv.) + SAS-to-U.3 cables. PCIe lane routing verification required per SCM-001. Adds ~2hr assembly time per unit. | N/A (U.3 bays native to SYS-221GE-NR) |
| NETWORKING & ISOLATION |  |  |
| Primary NIC | 2x 10GbE RJ45 (onboard) | 2x 25GbE SFP28 (onboard) |
| Management | IPMI 2.0 / BMC | IPMI 2.0 / BMC with dedicated port |
| Network-Isolated Mode | BIOS-disabled NICs + physical cable disconnect | Keyed hardware network kill switch (LED indicator) + physical cable disconnect |
| Isolation Threat Model | Prevents accidental outbound traffic; not rated for TEMPEST or classified environments | Electromechanical NIC isolation; prevents accidental and unauthorized outbound traffic; not rated for TEMPEST |
| TEMPEST / Classified | Custom engagement only | Custom engagement only (contact sales) |
| Kill Switch Vendor | N/A | Custom integration; part number, test protocol, and warranty terms documented in SCM-001 |
| POWER & THERMAL |  |  |
| Estimated Draw | ~650W peak / ~400W typical | ~1,400W peak / ~900W typical |
| PSU | 1x 1300W 80+ Platinum | 2x 2000W 80+ Titanium (1+1 redundant) |
| Cooling | Internal fans, standard office environment | Hot-swap redundant fans, server room recommended |
| Operating Temp | 10-35°C (50-95°F) | 10-35°C (50-95°F) |
| Noise Level | ~35 dBA (office-safe) | ~55 dBA (server room) |
| SOFTWARE & CONFIGURATION |  |  |
| Operating System | Allodial OS (Linux-based) | Allodial OS (Linux-based) |
| Included Software | Allodial OS, Centralized Ontology Engine, Multi-Model Orchestrator, Integration Layer | Allodial OS, Centralized Ontology Engine, Multi-Model Orchestrator, Integration Layer |
| Supported Config Matrix | See SCM-001 (DRAFT REQUIRED before pilot) | See SCM-001 (DRAFT REQUIRED before pilot) |
| SCM-001 Covers | BIOS version, GPU driver, OS kernel, Allodial OS version, ontology engine version | BIOS version, GPU driver, OS kernel, Allodial OS version, ontology engine version |
| Update Policy | Auto-updates disabled by default; managed updates via SMA | Auto-updates disabled by default; managed updates via SMA |

## BILL OF MATERIALS (HARDWARE)

Public distributor pricing (CDW, Newegg, ServerSupply, ASA Computers) as of April 2026. Software is bundled into the sale price and does not appear as a separate line item.

| COMPONENT | SENTINEL | QTY | BASTION | QTY |
| --- | --- | --- | --- | --- |
| Chassis (barebone) | $3,200 | 1 | $4,800 | 1 |
| GPU | $6,800 | 1 | $7,500 | 2 |
| CPU | $1,100 | 1 | $2,900 | 1 |
| RAM (per module) | $180 | 6 | $320 | 8 |
| OS SSD (960GB NVMe M.2) | $140 | 1 | $140 | 1 |
| Data SSD (3.84TB NVMe U.3) | $420 | 2 | $420 | 4 |
| U.3 AOC adapter + cable kit (Sentinel only) | $150 | 1 | - | - |
| PSU (included or upgrade) | $0 | - | $400 | 1 |
| Network kill switch (custom) | - | - | $300 | 1 |
| Assembly, burn-in, QA | $1,000 | - | $1,200 | - |
| HARDWARE BOM TOTAL | $14,310 |  | $28,980 |  |

## PRICING & SMA

All prices include Allodial OS and Centralized Ontology Engine. No separate software license fee.

| PRICING | SENTINEL | BASTION |
| --- | --- | --- |
| Hardware Sale Price (incl. software) | $24,900 | $49,900 |
| Hardware Gross Margin | 43% ($10,590) | 42% ($20,920) |
| SMA - Remote (mandatory Year 1) | $2,400/yr ($200/mo) | $5,500/yr ($458/mo) |
| Remote SMA Includes | Software updates, remote diagnostics, ship-to-depot hardware, 8x5 response | Software updates, remote diagnostics, ship-to-depot hardware, 8x5 response |
| Total Year 1 Customer Cost | $27,300 | $55,400 |
| Y1 Gross Margin | $12,990 (48%) | $26,420 (48%) |

## MARGIN SCENARIOS: PUBLIC vs. CHANNEL PRICING

Worst case = current public distributor pricing (no VAR agreements). Best case = negotiated channel/volume (15-20% GPU, 10-15% chassis reduction). No signed VAR agreements in place.

| SCENARIO | SENTINEL | BASTION |
| --- | --- | --- |
| WORST CASE (public pricing) |  |  |
| Hardware BOM | $14,310 | $28,980 |
| HW Gross Margin | 43% ($10,590) | 42% ($20,920) |
| Y1 Blended (w/ Remote SMA) | 49% ($13,340) | 48% ($26,420) |
| BEST CASE (channel pricing) |  |  |
| Hardware BOM (est.) | $12,720 | $25,760 |
| HW Gross Margin | 49% ($12,180) | 48% ($24,145) |
| Y1 Blended (w/ Remote SMA) | 53% ($14,580) | 54% ($29,645) |

## MARGIN JUSTIFICATION

Hardware gross margins of 42-43% at public pricing reflect the integrated appliance model. Bastion priced at $49,900 to sit below common $50K auto-approval thresholds in mid-market procurement; this trades ~$7K margin for 2-4 weeks faster sales cycles: Allodial OS, Centralized Ontology Engine, Multi-Model Orchestrator, and Integration Layer are bundled into the sale price with no separate software license. This is not a whitebox server with an OS installer. The software stack is the product; the hardware is the delivery mechanism. Year 1 blended margins of 48% (with mandatory remote SMA) fund ongoing R&D on the ontology engine and model orchestration layer, which constitute the competitive moat. Comparable enterprise AI appliances (Dell Enterprise Hub, NVIDIA DGX Station) carry 35-60% blended margins before service contracts.

CRITICAL: Gross margins shown above exclude assembly labor overhead, shipping, insurance, warranty reserves, and sales commission, which typically consume 8-12% of revenue in hardware startups. Real-world Y1 net margins will sit closer to 37-40%. All R&D and headcount planning MUST use worst-case public pricing at 35% net margin ceiling. Channel pricing is upside, not baseline.

## COMPANION DOCUMENTS

SCM-001: Supported Configuration Matrix - BIOS, GPU driver, OS kernel, Allodial OS, and ontology engine version locks. DRAFT REQUIRED BEFORE FIRST PILOT. Customer-initiated updates outside SCM void support eligibility.

SRC-001: Site Readiness Checklist - Power, cooling, network, rack space, and physical security requirements. Required as LOI Exhibit B.

SPS-001: Spares & Field Service Strategy - Third-party field service partner agreement (Park Place Technologies or Service Express) required for On-Site Premium SMA delivery. NOT YET IN MOTION.

Patent Status: Provisional patent filing — [DATE PENDING]. Do not use 'patent-pending' language in any materials until filing is confirmed.

CAVEATS: All BOM pricing is public distributor retail as of April 2026. GPU pricing is volatile. Model parameter estimates assume GPTQ/AWQ Q4 with VRAM reserved for KV cache at production context lengths. Bastion 70B tensor-parallel requires vLLM or TensorRT-LLM across 2x L40S via PCIe Gen 5; throughput limited to ~2-5 tok/s (no NVLink). Interactive inference at 70B requires NVLink-class hardware not in V1 product line. Fine-tuning estimates from community benchmarks, pending lab validation. Sentinel U.3 storage via PCIe AOC adapter requires lane routing verification per SCM-001. Network-Isolated Mode prevents accidental outbound traffic; not TEMPEST-rated. Bastion kill switch is a custom integration per SCM-001. On-Site SMA not available until SPS-001 is executed with a qualified field service partner; Remote SMA only for pilot. Channel pricing best-case assumes 17.5% GPU and 12.5% chassis reduction via unsigned VAR agreements.